NAME: VIJAYALAKSHMI S

REG NO: 192472326

COURSE CODE: CSA1216

COURSE NAME: COMPUTER ARCHITECTURE

ASSESSMENT TOOL : CO2 AT2

Q1. Signed Integer Addition Using 2's Complement

1. Identification of Errors / Bugs – 20%

For an 8-bit signed integer, the valid range is:

-128 to +127

Example:

120 + 50 = 170

But 170 cannot be represented using an 8-bit signed integer. If it is stored directly, the binary result wraps around and gives an incorrect negative value.

Bug: Overflow is not detected.

2. Debugging Approach & Analysis – 20%

For signed addition, overflow occurs when:

- Two positive numbers produce a negative result.

- Two negative numbers produce a positive result.

Another safe approach is to perform the calculation using a larger integer and check whether the result is within the 8-bit range.

3. Correctness of Debugged Solution – 25%

Python Code

4. Optimization Techniques Applied – 20%

- Use a larger integer temporarily.

- Check the range before storing the result.

- Avoid unnecessary binary conversions.

- Overflow detection is performed in constant time: O(1)

4) Testing and Documentation

| A | B | Expected |
| --- | --- | --- |
| 50 | 30 | 80 |
| 120 | 50 | Overflow |
| -100 | -40 | Overflow |
| -50 | 20 | -30 |
| 127 | 1 | Overflow |

Conclusion

The program correctly identifies when the result exceeds the 8-bit signed range and prevents an incorrect result.

Q2. Ripple Carry Adder → Carry Look-Ahead Adder

1. Identification of Errors / Bugs – 20%

In a Ripple Carry Adder, every bit must wait for the carry from the previous bit.

For example:

Bit 0 → Bit 1 → Bit 2 → Bit 3

This creates propagation delay.

Problem: Slow carry propagation.

2. Debugging Approach & Analysis – 20%

For each bit, calculate:

Pi = Ai XOR Bi

Gi = Ai AND Bi

where:

- Pi = Propagate

- Gi = Generate

The carry can then be calculated directly instead of waiting for the previous bit.

3. Correctness of Debugged Solution – 25%

Python Code and output

4. Optimization Techniques Applied – 20%

Ripple Carry:

Carry → Carry → Carry → Carry

CLA:

Carries calculated directly

Therefore, CLA reduces carry propagation delay and is faster for high-speed processors.

5. Testing and Documentation – 15%

| A | B | Expected Sum |
| --- | --- | --- |
| 3 | 2 | 5 |
| 7 | 5 | 12 |
| 8 | 7 | 15 |
| 15 | 1 | 16 |

Conclusion

Carry Look-Ahead Addition improves arithmetic performance by calculating carries in advance rather than allowing them to ripple through every bit.

Q3. Booth's Signed Multiplication

1. Identification of Errors / Bugs – 20%

Incorrect Booth multiplication generally occurs because of:

- Incorrect Q0 and Q-1 checking.

- Wrong arithmetic shift.

- Loss of sign bit.

- Incorrect subtraction for negative operands.

- Incorrect register width.

The Booth rules are:

| Q0 | Q-1 | Operation |
| --- | --- | --- |
| 0 | 0 | No operation |
| 0 | 1 | A = A + M |
| 1 | 0 | A = A - M |
| 1 | 1 | No operation |

2. Debugging Approach & Analysis – 20%

The algorithm maintains:

A = Accumulator

M = Multiplicand

Q = Multiplier

Q-1 = Previous Q bit

After every operation, perform an arithmetic right shift so that the sign bit is preserved.

3. Correctness of Debugged Solution – 25%

Python Code and output

4. Optimization Techniques Applied – 20%

Booth's algorithm reduces unnecessary additions/subtractions when consecutive multiplier bits are the same.

Instead of performing multiplication using repeated addition, it uses:

- Bit-pair encoding.

- Addition/subtraction.

- Arithmetic shifting.

This improves signed multiplication efficiency.

5. Testing and Documentation – 15%

Test:

5 × 6 = 30

-5 × 6 = -30

5 × -6 = -30

-5 × -6 = 30

0 × 10 = 0

Conclusion

The corrected Booth algorithm handles both positive and negative operands correctly by maintaining proper Q0, Q-1, and sign extension during arithmetic shifting.

Q4. Non-Restoring Division

1. Identification of Errors / Bugs – 20%

In restoring division, whenever the partial remainder becomes negative, it is restored by adding the divisor again.

This causes:

Subtract → Negative → Restore

Repeated restoration creates unnecessary operations.

2. Debugging Approach & Analysis – 20%

Non-restoring division avoids immediate restoration.

The rule is:

If previous A ≥ 0:

A = A - M

If previous A < 0:

A = A + M

Then the quotient bit is determined based on the sign of the new remainder.

3. Correctness of Debugged Solution – 25%

Python Code

4. Optimization Techniques Applied – 20%

The optimized method is:

Restoring Division

Subtract

↓

If negative

↓

Restore

Non-Restoring Division

Subtract/Add

↓

Continue

↓

Correct only when required

Thus, unnecessary restoration operations are reduced.

5. Testing and Documentation – 15%

| Dividend | Divisor | Quotient | Remainder |
| --- | --- | --- | --- |
| 20 | 4 | 5 | 0 |
| 25 | 5 | 5 | 0 |
| 17 | 4 | 4 | 1 |
| 10 | 3 | 3 | 1 |

Conclusion

Non-restoring division improves performance by avoiding repeated restoration of the partial remainder.

Q5. IEEE 754 Single Precision Floating Point

1. Identification of Errors / Bugs – 20%

IEEE 754 single precision contains:

Sign     = 1 bit

Exponent = 8 bits

Fraction = 23 bits

Total    = 32 bits

A float has limited precision. When adding a very large and very small number, the smaller number may have little or no effect because of exponent alignment and rounding.

Example:

100000000 + 0.00000001

The small value can be lost in single precision.

2. Debugging Approach & Analysis – 20%

The main causes are:

- Exponent alignment.

- Limited 23-bit fraction.

- Rounding.

- Loss of significant digits.

To improve accuracy, use double precision, which provides a larger significand.

3. Correctness of Debugged Solution – 25%

Python Code and output

4. Optimization Techniques Applied – 20%

Original

Python

Run

float

Optimized

Python

Run

double precision

Other techniques include:

- Avoiding unnecessary conversions.

- Using numerically stable algorithms.

- Using higher precision when required.

- Applying suitable rounding methods.

5. Testing and Documentation – 15%

| Large Value | Small Value | Purpose |
| --- | --- | --- |
| 1e8 | 1e-8 | Precision loss |
| 1e5 | 1e-5 | Exponent difference |
| 1.0 | 1e-8 | Small-value addition |
| 1e10 | 1e-10 | Large exponent difference |

Conclusion

Single precision has limited accuracy because of its 23-bit fraction. Double precision provides greater precision and is therefore preferred when scientific calculations require accurate results.