ANNEXURE A (ASSESSMENT TOOL 1)

APPLICATION BASED QUESTIONS

NAME: VIJAYALAKSHMI S

REG NO: 192472326

QUESTION 1

A smart city surveillance system

1. Identify

- Continuous video data is transferred between I/O devices, Memory, and CPU.

- During peak hours, data traffic increases.

- CPU waits for memory and I/O operations, causing delays in threat detection.

2. Analyze

The CPU processes instructions while memory stores data and I/O devices continuously send video frames. In a single-bus architecture, all components share one communication path, creating congestion. This increases waiting time and reduces processing speed. A multi-bus architecture allows simultaneous communication, reducing delays and improving system performance.

3. Compare

| Feature | Single Bus | Multi Bus |
| --- | --- | --- |
| Data Transfer | One transfer at a time | Multiple transfers simultaneously |
| Speed | Slow | Fast |
| Cost | Low | High |
| Performance | Lower | Higher |
| Traffic Congestion | High | Low |

4. Calculate (If Applicable)

Assume:

- CPU Execution = 4 ms

- Memory Access = 5 ms

- I/O Transfer = 6 ms

Single Bus

Total Time = 4 + 5 + 6

= 15 ms

Multi Bus

Memory and I/O occur simultaneously.

Total Time = 6 + 4

= 10 ms

Performance Improvement

15-1015×100=33.3%

5. Evaluate / Recommend

- Use Multi-Bus Architecture

- Implement Cache Memory

- Use DMA (Direct Memory Access)

- Apply Instruction Pipelining

- Increase CPU clock speed

6. Justify

Multi-bus architecture reduces communication bottlenecks. Cache memory minimizes memory access delays, and pipelining improves instruction throughput. Together they increase system responsiveness.

QUESTION 2

Real-time Stock Trading Application

1. Identify

- High CPI

- Frequent memory access

- Branch instructions

- Slow transaction processing

2. Analyze

The CPU repeatedly performs Fetch → Decode → Execute cycles. Frequent memory accesses and branch instructions cause pipeline stalls, increasing CPI and execution time.

3. Compare

| Without Optimization | With Optimization |
| --- | --- |
| High CPI | Low CPI |
| More stalls | Fewer stalls |
| Slow execution | Faster execution |
| Low throughput | High throughput |

4. Calculate

Formula

CPU Time=Instructions×CPI×Clock Cycle

Example

Instructions = 5 Million

Clock Cycle = 2 ns

Before

CPI = 4

CPU Time

= 5×10⁶ ×4×2 ns

= 40 ms

After

CPI = 2

CPU Time

= 5×10⁶ ×2×2 ns

= 20 ms

Performance Improved = 50%

5. Evaluate / Recommend

- Cache Memory

- Branch Prediction

- Instruction Pipelining

- Out-of-order Execution

- Prefetching

6. Justify

Reducing CPI directly lowers CPU execution time, improving transaction speed and system efficiency.

QUESTION 3

8085 Temperature Control System

1. Identify

- Incorrect addressing mode

- Wrong sensor readings

- Incorrect actuator response

2. Analyze

Different addressing modes determine how data is accessed. Choosing the wrong mode accesses incorrect memory locations, resulting in incorrect temperature processing.

3. Compare

| Addressing Mode | Description | Example |
| --- | --- | --- |
| Immediate | Data inside instruction | MVI A,25H |
| Direct | Address specified | LDA 2050H |
| Register | Uses register | MOV A,B |
| Register Indirect | Address stored in register pair | MOV A,M |

4. Calculate

Sensor Temperature

55°C

Stored at Memory Location

2050H

Correct Instruction

LDA 2050H

Wrong Instruction

MVI A,55H

Incorrect data may be loaded if actual sensor value changes.

5. Evaluate / Recommend

- Use Direct Addressing for sensor values

- Validate memory locations

- Test instructions before deployment

6. Justify

Correct addressing modes ensure accurate sensor readings and reliable industrial automation.

QUESTION 4

8086 Segmented Memory

1. Identify

- Stack overflow

- Incorrect data access

- Segment register errors

2. Analyze

8086 divides memory into:

- Code Segment (CS)

- Data Segment (DS)

- Stack Segment (SS)

- Extra Segment (ES)

Improper initialization causes invalid memory access and stack overflow.

3. Compare

| Segment | Purpose |
| --- | --- |
| CS | Stores instructions |
| DS | Stores variables |
| SS | Stores stack |
| ES | Stores extra data |

4. Calculate

Physical Address Formula

Physical Address

= Segment ×16 + Offset

Example

Segment = 2500H

Offset = 1000H

Physical Address

2500H ×16

=25000H

25000H+1000H

=26000H

5. Evaluate / Recommend

- Properly initialize CS, DS, SS

- Allocate sufficient stack space

- Use correct addressing modes

- Validate segment registers

6. Justify

Proper segmentation prevents memory corruption and ensures reliable multitasking.

QUESTION 5

Hexadecimal and Binary Conversion

1. Identify

- Wrong hexadecimal interpretation

- Incorrect binary conversion

- Incorrect instruction execution

2. Analyze

The CPU processes binary data internally. Incorrect hexadecimal conversion changes instruction values, causing processing errors.

3. Compare

| Hexadecimal | Binary |
| --- | --- |
| 0–F | 0000–1111 |
| Compact | Larger representation |
| Easy debugging | CPU processing format |

4. Calculate

Convert

3AH

Binary

3 =0011

A =1010

3AH

=00111010

Convert

5FH

Binary

5=0101

F=1111

5FH

=01011111

5. Evaluate / Recommend

- Use automatic conversion tools

- Verify hexadecimal values before execution

- Benchmark CPU performance

- Perform error checking

6. Justify

Accurate number conversion prevents execution errors and improves debugging efficiency in real-time systems.